Here is the compiled, self-contained project specification ready for Antigravity 2.0.

You can copy the raw markdown directly from the block below or run the quick shell one-liner beneath it to write it straight to your disk.

### File Content: docs/PROJECT_CONTEXT.md

Markdown

# Antigravity 2.0 Project Blueprint: Tiny Tapeout (Open-Source RTL-to-GDSII)

## 1. Project Overview & Target Constraints
- **Target Platform:** Tiny Tapeout Shuttle (SkyWater 130nm / sky130A or IHP SG13G2).
- **Physical Footprint:** Standard 1x1 tile (~300–800 standard cells).
- **Pinout Allocation:** Fixed 24-pin harness:
  - `ui_in [7:0]`: Dedicated digital inputs.
  - `uo_out [7:0]`: Dedicated digital outputs.
  - `uio_in [7:0]`: Bidirectional input path.
  - `uio_out [7:0]`: Bidirectional output path.
  - `uio_oe [7:0]`: Active-high output enable per bidirectional pin.
  - `clk`, `rst_n`, `ena`: System clock, active-low synchronous reset, chip power/enable.
- **Backend Toolchain:** LibreLane / OpenLane 2 containerized flow (Yosys, OpenROAD, OpenSTA, Magic, KLayout).
- **Primary Orchestrator:** Google Antigravity 2.0.

---

## 2. Recommended Directory Architecture

```text
tinytapeout-chip/
├── .antigravity/                   # Antigravity workspace settings
│   ├── project.json                # Project blueprints and default agent roles
│   └── mcp_servers.json            # Local MCP server bindings for EDA runners
│
├── .agents/                        # Agent execution contracts and skills
│   ├── rules/
│   │   ├── 01_system_limits.md     # Fixed pinouts, tile cell budget, reset conventions
│   │   └── 02_coding_style.md      # Verilog-2001, no latches, no internal tri-state
│   ├── skills/
│   │   ├── run_sim.py              # Executes Cocotb, extracts concise PASS/FAIL + diffs
│   │   ├── run_synth_check.py      # Quick Yosys pass, checks gate count vs budget
│   │   └── run_flow_signoff.py     # OpenLane 2 container gate, checks DRC/LVS/timing
│   └── roles/
│       ├── architect.md            # Microarchitecture planning & contract specifications
│       ├── test_engineer.md        # Cocotb test coverage & stimulus generator
│       └── backend_tuner.md        # OpenLane config, clock period & density tuner
│
├── src/                            # RTL Implementation
│   ├── tt_um_top.v                 # IMMUTABLE: Standard Tiny Tapeout top wrapper
│   └── my_core/                    # AGENT EDIT ZONE: Modular submodules live here
│       ├── datapath.v
│       └── control_fsm.v
│
├── test/                           # Verification Suite
│   ├── Makefile                    # Standard Cocotb test runner
│   ├── test_top.py                 # Primary Cocotb test suite
│   └── test_submodules/            # Modular unit tests
│
├── flow/                           # OpenLane / Backend Configs
│   ├── config.json                 # OpenLane 2 / LibreLane configuration
│   └── pin_order.cfg               # Preset pin constraints for Tiny Tapeout
│
├── scripts/                        # EDA Output Parsers (Keep agent contexts clean)
│   ├── parse_yosys_stat.py         # Extracts stdcell instance counts
│   └── parse_openlane_reports.py   # Extracts WNS, TNS, DRC, and antenna violations
│
├── docs/                           # Architecture specifications & state diagrams
│   └── PROJECT_CONTEXT.md          # This context blueprint
├── info.yaml                       # Tiny Tapeout metadata & pin documentation
└── Makefile                        # Top-level helper targets (sim, synth, harden)

3. Subagent Roles & Separation of Concerns

Lead Architect (High-Reasoning Model):

Decomposes target specs into microarchitectural state machines and datapath modules.

Manages interface contracts under docs/ and verifies module boundaries.

Strictly enforces the immutability of src/tt_um_top.v.

RTL & Logic Coder (Fast Coding Model):

Implements Verilog modules exclusively under src/my_core/.

Strictly adheres to Verilog-2001 standards.

Prohibited from creating asynchronous latches or modifying top-level wrapper pins.

Verification / Test Engineer (Fast Coding Model):

Implements functional testbenches in test/test_top.py using Cocotb.

  - Tests edge cases, clock gating, and reset behavior.

  - Requires 100% test pass status before signaling backend implementation.

- Backend Flow Tuner (Fast / Scripting Model):

  - Operates strictly on flow/config.json.

  - Adjusts placement density, core utilization, and target clock periods to clear DRC/timing violations without modifying core RTL.

## 4. Hardware Guardrails & Silicon Rules

- Immutability of Top-Level Wrapper: src/tt_um_top.v port names and bit widths must never be changed. All user logic must be instantiated inside src/my_core/.

- Synchronous Design Only:

  - All registers must be clocked on posedge clk.

  - Use active-low synchronous reset (rst_n).

  - No internal clock dividers or gated clocks; use clock enables (ena) instead.

- No Unintended Latches: Always specify default outputs in combinatorial always @(*) blocks and complete case statements with a default arm.

- Bus Integrity: Internal tri-state (z) assignments are forbidden. Tri-state is only supported on external pins via uio_oe.

- Resource Constraints: Stay within ~300 to 800 standard cells for a 1x1 tile footprint.

- Log Hygiene: Never feed raw EDA stdout logs to an agent. Agents interact with EDA tools strictly through wrapper scripts (run_sim.py, run_synth_check.py, run_flow_signoff.py) that filter outputs to actionable error traces under 40 lines.

## 5. Verification Gate Sequence

[Requirement / Feature Spec]
             │
             ▼
[Architect drafts microarchitecture in docs/]
             │
             ▼
[Coder implements Verilog in src/my_core/]
             │
             ▼
[Gate 1: Functional Sim] ──► python .agents/skills/run_sim.py
   ├── FAIL: Agent inspects error trace & fixes RTL
   └── PASS ──┐
              ▼
[Gate 2: Synth Check]    ──► python .agents/skills/run_synth_check.py
   ├── Exceeds Budget / Logic Optimized Out: Refactor RTL
   └── PASS ──┐
              ▼
[Gate 3: Backend Signoff] ──► python .agents/skills/run_flow_signoff.py
   ├── DRC / Timing Violation: Backend Tuner modifies flow/config.json
   └── CLEAN (DRC = 0, WNS >= 0): Tapeout Ready

## 6. Execution Rules for Antigravity

- Local Verification First: Never commit code or run backend synthesis without a passing run_sim.py run.

- Never Weaken Assertions: When tests fail, modify the hardware logic under src/my_core/—do not weaken test assertions unless the original specification is proven invalid.

- Visual Inspection Step: Once automated DRC/LVS passes, prompt the human engineer to visually inspect the final GDS layout in KLayout for power routing and pin alignment integrity.

---

### Quick Terminal Command to Save

To write this file directly to your project in one step from your terminal:

```bash
mkdir -p docs
cat << 'EOF' > docs/PROJECT_CONTEXT.md
# Antigravity 2.0 Project Blueprint: Tiny Tapeout (Open-Source RTL-to-GDSII)

## 1. Project Overview & Target Constraints
- **Target Platform:** Tiny Tapeout Shuttle (SkyWater 130nm / sky130A or IHP SG13G2).
- **Physical Footprint:** Standard 1x1 tile (~300–800 standard cells).
- **Pinout Allocation:** Fixed 24-pin harness:
  - `ui_in [7:0]`: Dedicated digital inputs.
  - `uo_out [7:0]`: Dedicated digital outputs.
  - `uio_in [7:0]`: Bidirectional input path.
  - `uio_out [7:0]`: Bidirectional output path.
  - `uio_oe [7:0]`: Active-high output enable per bidirectional pin.
  - `clk`, `rst_n`, `ena`: System clock, active-low synchronous reset, chip power/enable.
- **Backend Toolchain:** LibreLane / OpenLane 2 containerized flow (Yosys, OpenROAD, OpenSTA, Magic, KLayout).
- **Primary Orchestrator:** Google Antigravity 2.0.

---

## 2. Recommended Directory Architecture

```text
tinytapeout-chip/
├── .antigravity/                   # Antigravity workspace settings
│   ├── project.json                # Project blueprints and default agent roles
│   └── mcp_servers.json            # Local MCP server bindings for EDA runners
│
├── .agents/                        # Agent execution contracts and skills
│   ├── rules/
│   │   ├── 01_system_limits.md     # Fixed pinouts, tile cell budget, reset conventions
│   │   └── 02_coding_style.md      # Verilog-2001, no latches, no internal tri-state
│   ├── skills/
│   │   ├── run_sim.py              # Executes Cocotb, extracts concise PASS/FAIL + diffs
│   │   ├── run_synth_check.py      # Quick Yosys pass, checks gate count vs budget
│   │   └── run_flow_signoff.py     # OpenLane 2 container gate, checks DRC/LVS/timing
│   └── roles/
│       ├── architect.md            # Microarchitecture planning & contract specifications
│       ├── test_engineer.md        # Cocotb test coverage & stimulus generator
│       └── backend_tuner.md        # OpenLane config, clock period & density tuner
│
├── src/                            # RTL Implementation
│   ├── tt_um_top.v                 # IMMUTABLE: Standard Tiny Tapeout top wrapper
│   └── my_core/                    # AGENT EDIT ZONE: Modular submodules live here
│       ├── datapath.v
│       └── control_fsm.v
│
├── test/                           # Verification Suite
│   ├── Makefile                    # Standard Cocotb test runner
│   ├── test_top.py                 # Primary Cocotb test suite
│   └── test_submodules/            # Modular unit tests
│
├── flow/                           # Backend Implementation Files
│   ├── config.json                 # OpenLane 2 / LibreLane configuration
│   └── pin_order.cfg               # Preset pin constraints for Tiny Tapeout
│
├── scripts/                        # EDA Output Parsers (Keep agent contexts clean)
│   ├── parse_yosys_stat.py         # Extracts stdcell instance counts
│   └── parse_openlane_reports.py   # Extracts WNS, TNS, DRC, and antenna violations
│
├── docs/                           # Architecture specifications & state diagrams
│   └── PROJECT_CONTEXT.md          # This context blueprint
├── info.yaml                       # Tiny Tapeout metadata & pin documentation
└── Makefile                        # Top-level helper targets (sim, synth, harden)

## 3. Subagent Roles & Separation of Concerns

- Lead Architect (High-Reasoning Model):

  - Decomposes target specs into microarchitectural state machines and datapath modules.

  - Manages interface contracts under docs/ and verifies module boundaries.

  - Strictly enforces the immutability of src/tt_um_top.v.

- RTL & Logic Coder (Fast Coding Model):

  - Implements Verilog modules exclusively under src/my_core/.

  - Strictly adheres to Verilog-2001 standards.

  - Prohibited from creating asynchronous latches or modifying top-level wrapper pins.

- Verification / Test Engineer (Fast Coding Model):

  - Implements functional testbenches in test/test_top.py using Cocotb.

  - Tests edge cases, clock gating, and reset behavior.

  - Requires 100% test pass status before signaling backend implementation.

- Backend Flow Tuner (Fast / Scripting Model):

  - Operates strictly on flow/config.json.

  - Adjusts placement density, core utilization, and target clock periods to clear DRC/timing violations without modifying core RTL.

## 4. Hardware Guardrails & Silicon Rules

- Immutability of Top-Level Wrapper: src/tt_um_top.v port names and bit widths must never be changed. All user logic must be instantiated inside src/my_core/.

- Synchronous Design Only:

  - All registers must be clocked on posedge clk.

  - Use active-low synchronous reset (rst_n).

  - No internal clock dividers or gated clocks; use clock enables (ena) instead.

- No Unintended Latches: Always specify default outputs in combinatorial always @(*) blocks and complete case statements with a default arm.

- Bus Integrity: Internal tri-state (z) assignments are forbidden. Tri-state is only supported on external pins via uio_oe.

- Resource Constraints: Stay within ~300 to 800 standard cells for a 1x1 tile footprint.

- Log Hygiene: Never feed raw EDA stdout logs to an agent. Agents interact with EDA tools strictly through wrapper scripts (run_sim.py, run_synth_check.py, run_flow_signoff.py) that filter outputs to actionable error traces under 40 lines.

## 5. Verification Gate Sequence

[Requirement / Feature Spec]
             │
             ▼
[Architect drafts microarchitecture in docs/]
             │
             ▼
[Coder implements Verilog in src/my_core/]
             │
             ▼
[Gate 1: Functional Sim] ──► python .agents/skills/run_sim.py
   ├── FAIL: Agent inspects error trace & fixes RTL
   └── PASS ──┐
              ▼
[Gate 2: Synth Check]    ──► python .agents/skills/run_synth_check.py
   ├── Exceeds Budget / Logic Optimized Out: Refactor RTL
   └── PASS ──┐
              ▼
[Gate 3: Backend Signoff] ──► python .agents/skills/run_flow_signoff.py
   ├── DRC / Timing Violation: Backend Tuner modifies flow/config.json
   └── CLEAN (DRC = 0, WNS >= 0): Tapeout Ready

## 6. Execution Rules for Antigravity

- Local Verification First: Never commit code or run backend synthesis without a passing run_sim.py run.

- Never Weaken Assertions: When tests fail, modify the hardware logic under src/my_core/—do not weaken test assertions unless the original specification is proven invalid.

- Visual Inspection Step: Once automated DRC/LVS passes, prompt the human engineer to visually inspect the final GDS layout in KLayout for power routing and pin alignment integrity.
EOF

<FollowUp label="Want me to generate the parse_yosys_stat.py script next?" query="Generate the parse_yosys_stat.py and run_synth_check.py scripts for the .agents/skills directory."/>